For the servo amplifier system in the lab,

- The tachogenerator gain Kt= 2.717 mV/rpm

- The servo amplifier gain Kd= 7.4

- The operational amplifier gain Ka= 1

- The output potentiometer gain Ko= 4.5

- Gear Ratio=1/30

- Pre-amplifier gain Kp=10

Watch the videos of the speed control system as well as the attached figures in the folder "speed responses", then answer the following:

Question 1:

- Fill in the following table, show one sample of calculations.

|  | Attenuator at 8 |  | Attenuator at 7 |  | Attenuator at 6 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Open loop | Closed loop | Open loop | Closed loop | Open loop | Closed loop |
| Controller gain |  |  |  |  |  |  |
| Voss |  |  |  |  |  |  |
| τ |  |  |  |  |  |  |
| K |  |  |  |  |  |  |

- Compare between the open loop system and the closed loop system in terms of the time constant as the gain decreases.

- Find the transfer function of the motor for the closed loop response when the attenuator is at position 8. Show your work.

Question 2:

Watch the position control system videos and study the responses folder. If the figure name “second 9 9” means that both attenuators are at position 9,

- Fill in the following table. You have to assign the points on the graph for at least one case.

|  | Second 7 7 | Second 3 3 | Second 2 2 |
| --- | --- | --- | --- |
| The controller gain |  |  |  |
| The peak voltage Vp |  |  |  |
| The peak time Tp |  |  | --- |
| The steady state voltage Voss |  |  |  |
| The settling time Ts |  |  |  |
| Number of oscillations N |  |  |  |
| Percent overshot PO% |  |  |  |

- Calculate the Percent overshot for the three cases. Show one sample.

- Find the transfer function of the system for the case “second 7 7”. Show your work.

- What happened to the damping ratio as the system gain decreased?

- What happened when we operated the motor in its open loop format?

- Reconnect the circuit connected for the position control system in its closed loop configuration using the attached apparatus.